# Augmented Offense — Module 1: Fl3$hH00k (NTAG216 NDEF)

Len Noe & Dr. Gregory Carpenter · CW PENSEC

Phone · NFC Tools

# Module 1 — Fl3$hH00k: a URL that launches on tap, with zero interaction

Implant analog: xNT. Tool: your phone + NFC Tools. Fl3$hH00k is the tap that becomes a phish.

## Objective attendees reach

Write a URL NDEF record to an NTAG216 and watch an iPhone surface it with no app open and no button pressed. The victim did nothing.

## Materials

- Attendee phone (iPhone or Android) with NFC Tools installed

- 1× NTAG216 per attendee

- Optional Tier-1: pod laptop running module1/hook_server.py on a network you control

## Step by step (both platforms)

- Open NFC Tools → Write → Add a record → URL/URI.

- In the URL field, enter https://adversaryvillage.org/ (the default for this lab — a benign, real destination). (Advanced/Tier-1 only: instead use the local URL that hook_server.py prints, to demo the laptop callback.)

- Tap Write, then hold the NTAG216 to the phone.

- Re-tap to read it back and confirm the record is a URL type.

- Now tap it to an iPhone with no app open — iOS surfaces the link as a notification on its own. That passivity is the whole point.

### Tier 0 (everyone, zero install)

Open module1/index.html (hosted page or written to the tag). The page shows what it already knows about the visitor — user-agent, platform, screen, timezone — without asking. Point: you opened nothing, yet a page is profiling you.

### Tier 1 (optional, live callback — on your own laptop)

- Read module1/beacon.js and module1/hook_server.py first.

- cd module1 && python3 hook_server.py → it prints a URL.

- Write that URL to the NTAG216; uncomment the single Tier-1 line in index.html.

- Tap with your phone → your phone appears in the laptop log. That’s the callback.

## Why it matters

An implant (like an xNT) behaves identically to this card at the reader — the operator’s hand is the tag. A handshake becomes a phish; the operator doesn’t need a device, they are the device.

## Defensive angle

Treat tap-to-URL as untrusted input; apply MDM policy to background NDEF. The tap is the delivery, the trusted-looking prompt is the exploit — social-engineering amplified by hardware.

## Failure modes

| Symptom | Cause | Fix |
| --- | --- | --- |
| iPhone won’t auto-open the link | record not written as URL type | rewrite as a URL record |
| Reads are flaky | 13.56 MHz floor congestion | move off-pod, use the Faraday pouch to isolate, re-tap |
| Tier-1 phone can’t reach laptop | not on the same controlled network | put both on the pod hotspot |

## Safety

The default URL is the real, benign Adversary Village site (https://adversaryvillage.org/) — the point is the passive launch, not the destination. No credential harvesting. The optional Tier-1 beacon talks only to your own laptop. No attendee phone is ever a target of another attendee.

## Presenters

- Len Noe (213) — https://www.linkedin.com/in/len-noe/

- Dr. Gregory Carpenter (JunkBond) — https://www.linkedin.com/in/gcarpenter-cw-pensec/