# Augmented Offense — Module 3: H@nd$hak3 (T5577 125 kHz LF)

Len Noe & Dr. Gregory Carpenter · CW PENSEC

~19 min (1:04–1:23) · Flipper or Proxmark — no phone

# Module 3 — H@nd$hak3 (T5577 125 kHz LF), for complete beginners

You have never used a Proxmark or Flipper? Perfect — this assumes that. Follow it top to bottom. Do exactly what each numbered step says and check the green “✅ You should see” box before moving on. The “⚠️ If not” box tells you what to do when something’s wrong.

Implant analog: xEM / flexEM. Time: ~19 minutes.

No phone can do this module. 125 kHz has no radio in any phone. You need a Flipper Zero (works on its own) or a Proxmark3 (needs a laptop). We can hand you a Flipper if you don’t have a reader.

## 1. What are you about to do, in plain English?

The old 125 kHz “prox” fob is the round keyfob or thin white card that opens a lot of doors. It has no password — it just shouts a number, and the door believes it. That’s the entire security model, and it’s broken.

At the front of the room there is a master puck (213’s own retired door-strike key). You are going to read its number and copy it onto your own blank card. When you’re done, your card opens whatever the master opens — “you all now carry the master key.” That’s the point: copying a prox card is trivial and needs no secret.

You are copying 213’s provided master, on your own blank card. Never copy a real, in-use badge.

## 2. Mini-glossary

- EM4100 — the most common 125 kHz prox format. Its number is 10 hex characters (0–9, A–F).

- T5577 — a blank card you can write any prox number onto. That’s your target.

- Master puck — the physical card at the front you’ll copy from.

- Reader — your Flipper Zero (handheld, standalone) or Proxmark3 (board on a USB cable, needs a laptop).

## 3. What you need at your seat

- A Flipper Zero or a Proxmark3 + laptop (ask a pod lead if you need a Flipper)

- Your blank T5577 card

- Brief access to the master puck (shared at the front / passed around by a pod lead)

- The pod demo reader to test your finished card

## 4. Pick your path — DO ONLY ONE

This module has two paths. You do ONE of them, based on the reader in your hand. They are alternatives, NOT steps — do not do both.

- Holding a Flipper Zero? → do Path A, then stop. (Skip Path B entirely.)

- Holding a Proxmark3 + laptop? → skip Path A and do Path B.

Not sure which you have? The Flipper is a small handheld with a screen and a dolphin. The Proxmark is a bare circuit board on a USB cable. Ask a pod lead.

## Path A — Flipper Zero (skip this whole section if you’re using a Proxmark)

The Flipper is a handheld with a screen and arrow buttons. The 125 kHz antenna runs around the edges — the master puck and your T5577 sit flat on the back of the Flipper.

Buttons: center round button = OK/Select. Arrows move. Back = previous screen.

Menu wording varies slightly by firmware. If a label isn’t exact, pick the closest match — the flow is the same.

### A1. Open the 125 kHz app

- Press OK for the main menu.

- Arrow to 125 kHz RFID and press OK.

### A2. Read the master puck

- Choose Read.

- Hold the master puck flat against the back of the Flipper and keep it still.

✅ You should see a card read, showing a format like EM4100 and a number.

⚠️ If not: “Reading…” forever → the puck isn’t positioned right. Lay it flat centered on the back, don’t move it, and wait. Only one card on the reader at a time.

### A3. Save it

- Choose Save, name it master, confirm.

### A4. Write it onto your blank T5577

- From 125 kHz RFID → Saved, open master.

- Choose Write.

- Put your blank T5577 flat on the back of the Flipper and hold still.

✅ You should see “Successfully written” (or similar).

⚠️ If not: “Write failed” → the T5577 slipped. Re-center it on the back and try again. Make sure it’s the blank T5577, not the master.

### A5. Prove it worked

- From 125 kHz RFID, choose Read.

- Hold your T5577 to the back.

✅ You should see the same number as the master. Your card is now a copy. Tap it on the pod demo reader to confirm it reacts like the master. 🎉

🛑 STOP — you’re done with Module 3. Do NOT continue into Path B below; that’s the Proxmark version of the exact same thing. Move on to Module 4.

## Path B — Proxmark3 (the alternative to Path A — skip this if you already did Path A on a Flipper)

The Proxmark3 is a small board on a USB cable. The flat round coil is the LF (125 kHz) antenna — cards go there. Everything below is typed at the Proxmark console.

### B1. Open the console

- Plug the Proxmark3 into the laptop’s USB.

- In a terminal, type pm3 and press Enter. (If “not found,” try ./pm3 from the Proxmark folder, or ask a pod lead.)

✅ You should see a prompt:

[usb] pm3 -->

[usb] means it’s connected. Type every command below at this prompt.

⚠️ If not: [offline] → not connected. Replug the USB, try another port, run pm3 again.

### B2. Read the master puck (get its number)

- Lay the master puck flat on the round LF antenna.

- Type: lf search and press Enter.

✅ You should see it identify the card, e.g. EM410x ID found with a 10-hex number like 1A2B3C4D5E. Write that number down — it’s what you clone.

⚠️ If not: “No known 125 kHz tags found” → re-center the puck on the coil and run lf search again.

### B3. Write it onto your blank T5577

- Take the master off, lay your blank T5577 on the antenna.

- Type, replacing 1A2B3C4D5E with the number from B2:

lf em 410x clone --id 1A2B3C4D5E

✅ You should see a confirmation that it wrote the EM4100 ID to the T5577.

(If B2 reported HID instead of EM4100, use lf hid clone with the values it showed — ask a pod lead; most master pucks are EM4100.)

### B4. Prove it worked

- Keep the T5577 on the antenna. Type: lf search

✅ You should see the same number you wrote. That’s the clone confirmed. Tap it on the pod demo reader.

🛑 STOP — you’re done with Module 3. (Path A was the Flipper version of this same thing — you didn’t need it.) Move on to Module 4.

## 5. How do I know I succeeded?

Whichever one path you did ends the same way: read your T5577 and see the master’s number, then the pod demo reader reacts to your card the same as the master. If both are true, you cloned the master.

## 6. Why it matters

You just copied a door key by holding it near a $30 gadget — no password, no hack, no trace. Every low-frequency prox badge on Earth works exactly like this. An implant (like an xEM) is that copy: no card to carry — the operator walks up and they’re the key.

Defensive angle: physical access control that trusts a broadcast number has no defense here — the reader can’t tell a clone from the original, and it can’t tell a card from a hand. Retire 125 kHz prox.

## 7. Troubleshooting

| What you see | What it means | What to do |
| --- | --- | --- |
| “Reading…” never finishes | card not positioned | lay it flat/centered (Flipper back / Proxmark round coil), hold still, one card only |
| Proxmark: “No known 125 kHz tags” | reader can’t see it | re-center on the coil, retry lf search |
| Flipper: “Write failed” | T5577 slipped | re-center on the back, retry; confirm it’s the blank |
| Clone reads a different number | wrong ID typed (Proxmark) | re-check the number from lf search, redo the clone |
| Proxmark prompt [offline] | not connected | replug USB, other port, run pm3 again |
| Everyone’s card is identical | expected — you all copied one master | that IS the lesson: “you all carry the master key” |

## 8. Safety (non-negotiable)

- Copy only the provided master puck, onto your own blank T5577.

- Never read or clone anyone’s real building badge, keyfob, or hotel/transit card — not even “just to see.”

- These skills are illegal against systems you don’t own or aren’t authorized to test. Here, on the provided master and your own blank, you’re clear.

## Presenters

- Len Noe (213) — https://www.linkedin.com/in/len-noe/

- Dr. Gregory Carpenter (JunkBond) — https://www.linkedin.com/in/gcarpenter-cw-pensec/